# Test Procedure for EB-055-00-1

The following instructions show the test procedure for EB-055-00-1 USB E-Block Board.

Items required:

- 13.5V power supply (min 13.25V)

- 1 x Multi programmer board (EB-006-00-1)

      - With PIC18F4455 inserted in 40 pin DIL (instead of usual 16F88)

- 1 x LCD board (EB-005-00-1)

- Computer with PPP v3 installed that has a USB connection

- USB cable

- Wire

## Set-up

- Ensure that the Multiprogrammer is in correct configuration

  - XTAL mode (SW1 towards the centre of the board)

  - Ensure that a 4MHz crystal is inserted in the Multiprogrammer board

- Insert the LCD board (EB-005-00-1) into Port B of the Multiprogrammer

- Connect wire from “+V” of LCD board to “+V” of Multiprogrammer

- Program the a PIC18F4455 with the test program USB.hex

## TEST

- Insert the USB board into Port C of the Multiprogrammer

- Insert the USB cable into the socket on the USB board

- Connect wire from “+V” of the USB board to “+V” of Multiprogrammer.

- Make sure 3-way jumper is in position "A" on the USB board.

- Switch on Power to on the Multiprogrammer.

- The word “Ready: ” should appear on the LCD screen.

- Open Hyperterminal on the PC by clicking on Start -> Run and typing in hypertrm

- Enter a name for the new connection  and click OK

- Under the Connect Using Option select the COM port relating to the USB board and click OK.

- Select a Bits per second rate of 9600 and change the Flow control to None then click Ok.

- Type some data into the hyperterminal window and the data should be displayed on the LCD.

- Close Hyperterminal

If the data displayed on the LCD then the board passed the test.

If no data was displayed then the board failed the test.