EB070-12-1 USBee Factory Test Document

The following instructions describe the test procedure for the EB070-00-1 USBee board.

- System Setup

Multi-programmer board (EB-006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F877A |
| SW1 (Fast/Slow) | Don’t care |
| SW2 (RC/Xtal) | Xtal |
| Xtal frequency | 19.6608MHz |
| Port A |  |
| Port B | LCD Board EB005 |
| Port C |  |
| Port D | USBee Board EB070 |
| Port E |  |
| Additional | LED Board EB004 |
| Test program | USBee.HEX |

## 2.   Test procedure

- Install the USBee SX bundle from the Matrix Multimedia USBee Product Page

- Install the USBee Suite bundle from the Matrix Multimedia USBee Product Page

- Connect EB005 (LCD board) to PORTB of the Multiprogrammer.

- Connect EB070 (USBee board) to PORTD of the Multiprogrammer.

- Connect EB004 (LED board) to the downstream port of the USBee board.

- Ensure EB005 jumper setting is in the default position.

- Connect USB cable to computer and EB006.

- Connect USB cable to computer and EB070.

- Connect a wire from the +V screw terminal of the Multi-programmer to +V terminal of LCD board.

- Ensure 13.5V power supply is working correctly and connect to EB006.

- Open PPP.

- In PPP open file USBee.HEX.

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- The LCD will confirm the EB006 is running correctly by printing the message “Test Running”

- Run the USBee Suite software and ensure the button at the bottom says “Capture Once” and not “Demo”.

- If the button says demo then ensure USB software has been installed correctly and that USBee is connected to the PC via USB.

- Click the Speed And Samples menu, set the Sample Rate to 8Msps and the Buffer Size to 2M samples.

- Click the “Capture Once” button and check that the output is as shown below, note you may have to use the mouse wheel to scroll down and zoom out.

- Check that the LEDs on the LED board all light when the test is running.

If the USBee Suite is collecting data as shown on the next page and all the LEDs are lighting on the EB004 then the board has passed the testing.

Sample output from successful test