Toggle Switch factory test routine

EB-074-12-1

# Testing this product

### 1	What is required and Hardware Setup

| Hardware | Setting |
| --- | --- |
| Multi programmer board (EB-006) | With PIC16F877A inserted in 40 pin DIL (instead of usual 16F88) and set SW2 to XTAL |
| Power supply | External, 13.5V, (Powered with a current-limited supply (about 150) mA. to prevent accidental damage , apply to Multi programmer board (EB-006) |
| Computer with PPP v3 with a USB cable | Connect USB cable between Multi programmer board and PC |
| LED board (EB-004) | Connect to PORT C of Multi programmer board (EB-006) |
| E-blocks IDC cable | Connect to PORT D of Multi programmer board |
| Wire | Connect to +V of Multi programmer board |
| Test program | eb074-14-1.HEX |

## 2	Software set up

- Open PPP v3

- Click Configure PICmicro..

- Click Autodetect PICmicro  to detect PIC automatically or you can select the drop down list for PIC16F877A, then click “OK”

- Click Open  to navigate the program eb074-14-1.HEX

- Click Send to PICmicro  to send the file to PIC and wait few seconds until the pop-up window telling the status is “Program sent and verified OK” then click “OK and ”press reset button on Multi programmer board.

## 3	Test Procedure

- Plug in Switch board – EB-074 (test subject) to E-blocks IDC cable

- Connect wire from screw terminal “+V” of Multi programmer board to the screw terminal “+V IN” on Switch board - EB-074 (test subject)

- Slide each of the eight switches SW0 to SW7

- The corresponding LED should illuminate, for example If SW0 is connected D0 will illuminate or If SW1 is connected D1 will illuminate and so on.

If all LEDs illuminate correspondingly to switches positions, then the test subject board passes the test. You can go further by sliding more than one switch, this should illuminate more than 1 Led e.g. sliding SW2 AND SW4 into the on position will illuminate D2 AND D4.

- Unplug Switch board (EB-074) and unscrew the wire in +V IN

- Repeat from step 1 for next test subject.