Sensor board factory test routine

EB-003-12-2

1.  About this document

This document concerns the Matrix Multimedia Sensor board code EB003-00-2.

- Trademarks and Copyright

PIC and PICmicro are registered trademarks of Arizona Microchip Inc.

E-blocks is a trademark of Matrix Multimedia Limited.

EB003 and associated software and documentation are Copyright ©2006 Matrix Multimedia Limited.

- Other Sources of Information

There are various other documents and sources that you may find useful:

- Disclaimer

The information provided within this document was correct at the time of going to press.  Matrix Multimedia reserves the right to change specifications from time to time.

- Technical Support

If you have any problems operating this product then please refer to the troubleshooting section of this document first.  You will find the latest software updates, FAQs and other information on our web site: www.matrixmultimedia.co.uk.  If you still have any problems please email us at: support@matrixmultimedia.co.uk.  When emailing please state the operating system and the version of PPP you are using.

# Testing this product

- Hardware Setup

Multi-programmer board (EB006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F877A |
| SW1 (Fast/Slow) | Don’t care |
| SW2 (RC/Xtal) | Xtal |
| Xtal frequency | 19.6608MHz (default) |
| Port A | EB003 Sensor board |
| Port B | EB005 LCD board |
| Port C |  |
| Port D |  |
| Port E |  |
| Test program | EB003-14-2.hex |

| EB003 Options | Setting |
| --- | --- |
| Patch jumper setting | DEFAULT |

| EB005 Options | Setting |
| --- | --- |
| Patch jumper setting | DEFAULT |

- Insert Vernier motion detector into EB003 J3

- Insert a Vernier analogue sensor into EB003 J4

- Wire power to both the EB003 and the EB005

- Note that you will find it easier if EB003 is on an IDC extension lead

## 2	Software set up

- Install PPP from the ELSAM CD ROM or from the Matrix Multimedia web site www.matrixmultimedia.com

- Select OPTIONS…CONFIGURE PICMICRO

- Select 16F877A from the pull down menu

- Select XTAL as the oscillator type

- Open the test program

- Click on the SEND to PICMICRO button

## 3	Test Procedure

- Press the reset button on the Multiprogrammer.

- Check the LCD display – the display should be as follows: -

- On EB003 cover up the on board light sensor

  - this should change the values on the 1st number on the LCD display

- Alter the position of EB003 RV1 pot

  - This should change the values on the 2nd number on the LCD display

- Monitor the condition of the analogue sensor

  - This should change when plugged in and represent temperature between 0 to 255.

- Alter the angle of the Motion detector

  - This will alter the 4th number on the LCD labelled “d = ”

  - Note that you should be able to hear the Motion detector ‘clicking’

  - Note that the Motion sensor seems to work best when the internal switch is in the Ball/Human position.

This should satisfy that the Sensor Board is fully functional!