EB-054 Pasco Sensor Board

Test Procedure

Requirements

- EB-054 Pasco Sensor Board to be tested

- EB-005 LCD Board

- EB-006 Multiprogrammer with PIC16F877A microcontroller containing the test program EB054-14-1.hex.

If the microcontroller does not contain the program EB054-14-1.hex, it can be downloaded from a PC using the Matrix Multimedia PPP programming utility.

- +13.5v Power supply for the EB-006

- -12v, 0v, +12v Power supply for the EB-054

Test System Setup

Connections

| EB-006 Multiprogrammer Port | E-Blocks Board |
| --- | --- |
| PORT A | EB054 test board connector |
| PORT B | Not used |
| PORT C | EB-005 LCD Board |
| PORT D | Not used |

Connect the LCD Board to PORT C of the Multiprogrammer.

Connect the +V terminal on the Multiprogrammer to the +V terminal on the  LCD board.

Connect a flying lead from a +V terminal on the Multiprogrammer to supply 5v to the Pasco Sensor Boards being tested.

Test System - board configurations

| EB-006 Multiprogrammer | Setting |
| --- | --- |
| SW1 | FAST |
| SW2 | XTAL |
| J12, J13, J14 | USB |
| J11, J16, J17 | I/O PORT |
| J29 | PSU |

| EB-005 LCD Board | Setting |
| --- | --- |
| J3/J4 | DEFAULT |

Connect the 13.5v power supply to the Multiprogrammer.

The test program will run immediately and will produce a 'FAILED' result with no Pasco Sensor Board connected.

Test Procedure

- Configure the EB-054 Pasco Sensor Board

| EB-054 Pasco Sensor Board |  | Setting |
| --- | --- | --- |
| J4/J5 |  | DEFAULT (A) |
| Analog A | Uni/Bi | Bi |
|  | 5V/12V | 12V |
| Analog B | Uni/Bi | Bi |
|  | 5V/12V | 5V |

- Connect the test lead

| Lead connector | Board connector |
| --- | --- |
| DIN connector A | Analog A |
| DIN connector B | Analog B |
| Jack plug C | Digital A |
| Jack plug D | Digital B |

- Connect the EB-054 Pasco Sensor Board to the test system

Plug the Pasco Sensor Board into the PORT A connector of the Multiprogrammer

- Connect the power supply leads

Connect the -12v, 0v and +12v supplies to the -12v, GND, and +12v terminals of the Pasco Sensor Board.

Connect the +V lead  from the Multiprogrammer board to the +5V terminal of the Pasco Sensor Board.

- Run the test program

Press the RESET button on the Multiprogrammer

The system will perform a series of tests on the Pasco Sensor Board and display a PASSED/FAILED result on the LCD.

Results

| PASSED | FAILED |
| --- | --- |
| The board has passed the tests. | The board has failed the tests. - Check all the power supply voltages at the board terminals. - Check all the board jumper settings. - Check that the test lead plugs are in the correct sockets and have been inserted properly. - If any faults were found, re-run the tests by pressing the RESET button on the Multiprogrammer board |
| Disconnect the power supply leads, taking care not to create any short-circuits. Disconnect the Pasco Sensor Board from the test system. Remove the test cable. Move the Analog B 5V/12V Jumper to 12V Store the board according to the PASSED/FAILED results of the test. |  |