# EB064 – Factory Test Routine

## System Setup

1 x Multi-programmer board (EB064) with:

| EB064 Options | Setting |
| --- | --- |
| Power supply | External, 8V |
| Power Supply Jumper (J1) | PSU |
| Programming Pins Jumper (J9) | A |
| I/O pin jumper (J18) | VRAIL |
| PICkit Jumper (J5) | USB |
| Xtal frequency | 12MHz |

## Test Procedure – Part 1

  - Setup the system as shown above.

  - Connect a USB cable between the Multiprogrammer and a Powered USB hub.

  - Switch the voltage of the PSU to 8V.

  - Insert a dsPIC30F4013 into the socket U1.

  - Check the I/O pin jumper J18 is In the VRAIL position

  - Power up the Multiprogrammer by connecting and switching on the power supply.

  - Load the mLoader software and check that the option Chip -> Write Config is ticked.

  - Using the mLoader software erase the dsPIC device.

  - Using the mLoader software send the Test pt1.hex program to the dsPIC device.

  - Connect a LED E-block (EB004) to the ports one at a time and check that the following LEDs light up.

|  | Bit0 | Bit1 | Bit2 | Bit3 | Bit4 | Bit5 | Bit6 | Bit7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PORTA | - | - | X | X | - | X | - | - |
| PORTBL | X | X | X | X | X | X | X | X |
| PORTBH | X | X | X | X | X | - | - | - |
| PORTD* | X | X | X | X | X | X | - | - |
| PORTF | X | X | X | X | X | X | X | - |

## *denotes that d4 and d5 are out of sequence

## Test Procedure – Part 2

  - Power down the Multiprogrammer by removing the power supply and USB connection.

  - Remove the PIC24FJ16GA002 device from the board.

  - Insert a dsPIC30F2011 into the socket U7.

  - Move the power supply jumper J1 from the PSU position to the USB position.

  - Move the programming pins jumper to the C position.

  - Move the I/O pins jumper J16 to the I/O position.

  - Connect a USB cable between the Multiprogrammer and a Powered USB hub.

  - Load the mLoader software and check that the option Chip -> Write Config is ticked.

  - Using the mLoader software erase the dsPIC30F2011 device.

  - Using the mLoader software send the EB064_B.hex program to the dsPIC30F2011 device.

  - Connect a LED E-block (EB004) to ports A, BL and D one at a time and check that the following LEDs light up. No two LEDs should be on at any one time.

|  | Bit0 | Bit1 | Bit2 | Bit3 | Bit4 | Bit5 | Bit6 | Bit7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PORTA | - | - | X | X | - | - | - | - |
| PORTBL | X | X | X | X | X | X | X | X |
| PORTD | X | - | - | - | - | - | - | - |

If test complete ok, all LEDs are lighting up and no more then 1 LED is on at any one time then the board has passed the test routine correctly.

If the microcontroller fails to ID or program correctly then check your board settings match those shown above.

If programming completes correctly but the LEDs fail to light then check that you have a 12MHz crystal inserted into the crystal socket and that it is making a good contact with the socket connections.

If you are still having problems then try using an alternate dsPIC2011 or PIC24FJ16GA002 device to test out the board connections to ensure it is not the microcontroller device itself that is damaged.