The following instructions describe the test procedure for the EB066-00-2 Quad Band GSM board.

- System Setup

Multi-programmer board (EB-006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F877A |
| SW2 (RC/Xtal) | XTAL |
| Xtal frequency | 19.6608MHz |
| Port B | LCD EB005 |
| Port C | GSM EB066 |
| Test program | GSM_Test.hex |

- Connect EB005 (LCD board) to PORTB of the Multiprogrammer.

- Connect USB cable to computer.

- Connect power supply to EB006 and switch on.

- Open PPP.

- In PPP open file GSM_Test.hex.

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen.

- Unplug power supply.

## 2.   Test procedure

- Connect EB066 (GSM board) to PORTC of the Multiprogrammer.

- Ensure that the GSM EB066 has the following jumper configuration:

  - J5 	= 	No Jumper

  - J6 	= 	4-Way Jumper Fitted

  - J7 	= 	No Jumper

  - J12 	= 	No Jumper

  - J13 	= 	2-Way Jumper Fitted

- Connect a wire from the +V screw terminal of the Multi-programmer to +V terminal of LCD board.

- Connect a wire from the +V screw terminal of the Multi-programmer to +V terminal of GSM board.

- Connect a wire from the +14V screw terminal of the Multi-programmer to VPWR terminal of GSM board.

- Insert a SIM card into the socket on the GSM Board.

- Connect power supply to EB006 and switch on.

- Press the Reset button on the EB006.

- The LCD should display Starting.

- The Blue LED should light up and start flashing at a rate of 1hz.

- The display should then display OK.

- After a few second the 1hz flashing should change to a 3 second cycle, 3 seconds off, 200ms on. This indicates the GSM module has established communications with the network provider

- Test passed.

- If nothing appears on the screen or the frequency of the LED flash does not alter or fails to light then the board has failed the test routine.