EB076-12-2 4D Picaso Touchscreen Display Factory Test Document

The following instructions describe the test procedure for the EB076-00-2 4D Picaso Touchscreen Display board.

- System Setup

Multi-programmer board (EB-006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F1937 |
| SW1 (Fast/Slow) | Don’t care |
| SW2 (RC/Xtal) | Xtal |
| Xtal frequency | 19.6608MHz |
| Port A |  |
| Port B | EB004 LED Board |
| Port C | EB076 4D Goldelox Display |
| Port D |  |
| Port E |  |
| Test program | EB076.HEX |

## 2.   Test procedure

- Connect EB076 (4D Picaso Touchscreen Display board) to PORTC of the Multiprogrammer.

- Connect EB004 (LED board) to PORTB of the Multiprogrammer.

- Ensure EB076 jumper settings are in the B position.

- Connect a wire from the +V screw terminal of the Multi-programmer to 5V terminal of 4D Display board.

- Connect USB cable to computer and EB006.

- Ensure 9V power supply is working correctly and connect to EB006.

- Open PPP.

- In PPP open file EB076.HEX when using the SIL connector display.

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen.

- The LED board will flash at 1 second intervals signifying the program on the EB006 is running correctly.

- The display should display a plain white background

- The display should print the text ”READY!” in Blue

- Touching the screen should draw a blue circle

- Dragging your finger across the screen should draw a blue line made up of circles.

If the LEDs are flashing at the 1 second rate and the 4D display is displaying the white background with blue lines where the display has been touched then the board has passed the test routine.