# Test Procedure for EB-021-00-1

The following instructions show the test procedure for EB-021-00-1 MIDI E-Block Board.

Items required:

- 2 x Multiprogrammer E-Blocks with 16F877A

- 1 x MIDI E-Block (unit under test)

- 1 x Pre-tested MIDI E-Block

- 1 x LCD Display E-Block

- 2 x MIDI cable

- 1 x Power Supply

- 1 x E-blocks female to male IDC Cable

## Set-up

- 16F877A in 40-way socket on both Multiprogrammers

- SW2 set to XTAL Mode on both Multiprogrammers

- 19.6608 MHz Crystals in both Multiprogrammers

- LCD Display E-Block connected to Port D on master Multiprogrammer

- LCD E-Block DEFAULT jumper setting

- Wire connection between +V on master Multiprogrammer to +V on LCD E-block

- Pre-tested MIDI E-block connected to Port C on master Multiprogrammer

- Connect +V of master Multiprogrammer to +V of pre tested MIDI E-block

- J11 of pre-tested MIDI E-block – jumper in +5V position

- J2 of pre-tested MIDI E-block – jumper in position A

- Connect a wire from the Master Multiprogrammers +14V to the Slave Multiprogrammers +14V

- Connect a wire from the Master Multiprogrammers GND to the Slave Multiprogrammers GND

- Connect the master Multiprogrammer to the power supply

- Connect MIDI cable A to the IN port of the pre-tested MIDI E-block

- Connect MIDI cable B to the OUT port of the pre-tested MIDI E-block

- Attach the E-Blocks IDC cable to Port C of the Slave Multiprogrammer

- Connect a wire from the +V of the slave Multiprogrammer

- Set up the configuration as specified above.

- Download the Master.hex file to the master Multiprogrammer with the pre-tested MIDI board and LCD E-Block connected and wired.

- Connect the master Multiprogrammer to the power supply

Diagram of full MIDI test configuration

## EB-021 MIDI Board Test Routine

- Connect the free end of the IDC cable to the MIDI unit under test

- Connect MIDI cable A to the OUT port of the MIDI E-block unit under test

- Connect MIDI cable B to the IN port of the MIDI E-block unit under test

- Connect the free end of the +V wire to the +V on the MIDI E-block under test

- Check that J11 of the MIDI E-block under test is set to 5V

- Check that J2 of the MIDI E-block under test is set to A

- Press reset on the slave Multiprogrammer

- Press reset on the master Multiprogrammer

- The Master will now test the Slave device by sending and receiving MIDI messages

- The Master will display “Test Successful” for a working MIDI E-Block.

- The Master will display “Test Failed” for a failed MIDI E-Block.

- Unplug all cables from the tested MIDI E-Block and remove the power supply from the slave Multiprogrammer.