Opto-isolator board factory test routine

EB-035-12-1

1.  About this document

This document concerns the Matrix Multimedia Optoisolator Board code EB035-00-1.

- Trademarks and Copyright

PIC and PICmicro are registered trademarks of Arizona Microchip Inc.

E-blocks is a trademark of Matrix Multimedia Limited.

EB035 and associated software and documentation are Copyright ©2006 Matrix Multimedia Limited.

- Other Sources of Information

There are various other documents and sources that you may find useful:

- Disclaimer

The information provided within this document was correct at the time of going to press.  Matrix Multimedia reserves the right to change specifications from time to time.

- Technical Support

If you have any problems operating this product then please refer to the troubleshooting section of this document first.  You will find the latest software updates, FAQs and other information on our web site: www.matrixmultimedia.co.uk.  If you still have any problems please email us at: support@matrixmultimedia.co.uk.  When emailing please state the operating system and the version of PPP you are using.

# Testing this product

The following instructions describe the test procedure for the EB035-00-1 Opto-isolator board.

- Hardware Setup

Multi-programmer board (EB006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F877A |
| SW1 (Fast/Slow) | Don’t care |
| SW2 (RC/Xtal) | Xtal |
| Xtal frequency | 19.6608MHz (default) |
| Port A |  |
| Port B |  |
| Port C | EB004 LED board |
| Port D | Opto-isolator board EB-035 |
| Port E |  |
| Test program | EB035-14-1.hex |

| EB035 Options | Setting |
| --- | --- |
| Patch jumper setting | LOW nibble |

Wire power to the opto-isolator board.

To test the opto-isolator board you will also need a pair of test probes with a DC voltage of 5V between the two probes.

## 2	Software set up

- Install PPP from the ELSAM CD ROM or from the Matrix Multimedia web site www.matrixmultimedia.com

- Select OPTIONS…CONFIGURE PICMICRO

- Select 16F877A from the pull down menu

- Select XTAL as the oscillator type

- Open the test program

- Click on the SEND to PICMICRO button

## 2	Test Procedure

- All LEDs on PORTC should be off.

- Using the test probes apply a voltage across each of the 4 input pairs in turn with a test probe.

- LEDs 0 to 3 on the Opto-isolator board should turn on, and the corresponding LED on the LED board on Port C should turn off.

- Change patch setting J6 / J7 to HIGH and repeat with the new patch setting.

| EB035 Options | Setting |
| --- | --- |
| Patch jumper setting | HIGH nibble |

.

- Again apply voltage to each terminal input in turn. The corresponding LED (D4 to D7) should switch on.

If all LEDs switch off when the corresponding switch is pressed for both LOW and HIGH nibble settings then the board has pass the test..