# EB039 – USB232 – User Test Routine

### Requirements

- PC with the following

  - HyperTerminal (or equivalent terminal emulator)

  - Spare USB port

- Hardware:

  - Multiprogrammer E-Block with pre-programmed 16F877

  - LCD display E-Block

  - Switch E-Block

  - USB232 E-Block (unit under test)

  - USB cable

  - Power supply

### Hardware setup

- Connect the LCD E-Block to port B and the Switch E-Block to port A of the Multiprogrammer E-Block.  The LCD should be in the “default” setting.

- Plug the EB039 E-Block into Port C of the Multiprogrammer.  Use jumper settings “C” and “1”.

- Connect +V power from Multiprogrammer to both the Switches and the LCD.

- Connect the 13V power supply to the Multiprogrammer.

- Make sure the clock switches are both to the right (XTAL and FAST).

### Software setup

- Plug the EB039 USB232 E-Block into the PC using a USB cable.  If this is the first time the device is being plugged in, the “Add new hardware” wizard will appear and you will need to install the appropriate drivers.  See the separate “Driver Installation” document for guidance.

- The first step is to determine which virtual COM port has been allocated to the device.  Do this by clicking on the “Start” button and selecting “Run…”.  In the “Open:” box, type “devmgmt.msc” (without the quotes) and press “OK” – this will open the “Device Manager” window.

- Locate the “Ports (COM & LPT)” section and expand it by clicking the “+” symbol.  The allocated COM port number will be shown in brackets after the “USB232 E-Block” entry.

- Close down the “Device Manager” window.

- Open “HyperTerminal”.  Enter a name in “Connection Description” window if this appears – it does not matter what you enter here.

- In the “Connect to…” window, select the appropriate COM port number from the “Connect using:” drop-down box.

- Set the properties for this connection to the following:

  - Bits per second = 9600

  - Data bits = 8

  - Parity = None

  - Stop bits = 1

  - Flow control = Hardware

- The connection should now be active.

### Unit test procedure

- Press the ‘reset’ button on the Multiprogrammer - the LCD display should read “EB039 test” and “Press A0”.

- Press A0 – HyperTerminal should display the message “EB039 Test” and the LCD display should display “Waiting for PC..”

- Type a few characters into the HyperTerminal window – whatever you typed should be displayed on the lower part of the LCD display (and not on the PC window).

- Press <return> on the PC keyboard and the LCD display should read “**Unit Passed**”.