EB072-12-1 EB006 ZIF add-on Factory Test Document

The following instructions describe the test procedure for the EB072-00-1 EB006 ZIF add-on board.

- System Setup

Multi-programmer board (EB-006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| SW1 (Fast/Slow) | Fast |
| SW2 (RC/Xtal) | RC |
| Xtal frequency | 19.6608MHz |

## 2.   Test procedure

- Connect USB cable to computer and EB006.

- Ensure no target chips are inserted in the EB006 multiprogrammer

- Connect EB072 ZIF adapter to DIL headers on the EB006 ensuring “this way up” arrow points towards the EB006 USB socket.

- Ensure 13.5V power supply is working correctly and connect to EB006.

- Open PPP.

- Insert the 16F636 chip into the 8/14/20-way ZIF socket ensuring it is in the correct position and orientation.

- Ensure the 4-way jumper is in the 8/14/20PIN position.

- In PPP open file 16F636.HEX

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- Remove 16F636 From the ZIF socket.

- Insert the 16F88 chip into the 18-way ZIF socket ensuring it is in the correct position and orientation.

- Ensure the 4-way jumper is in the 18PIN position.

- In PPP open file 16F88.HEX

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- Remove 16F88 From the ZIF socket.

- Insert the 16F870 chip into the 28-way ZIF socket ensuring it is in the correct position and orientation.

- Ensure the 4-way jumper is in the 28PIN position.

- In PPP open file 16F870.HEX

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- Remove 16F870 From the ZIF socket.

- Insert the 16F877A chip into the 40-way ZIF socket ensuring it is in the correct position and orientation.

- Ensure the 4-way jumper is in the 40PIN position.

- In PPP open file 16F877A.HEX

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- Remove 16F877A From the ZIF socket.

If the “Program Sent And Verified” message appears for all four chip types then the test routine has been passed correctly.