E-Block AVR Factory Test Routine

EB019-12-2

# Testing this product

- What is required and Hardware Setup

| Hardware | Setting |
| --- | --- |
| AVR board EB-019 (test subject) |  |
| 4 x LED boards EB-004 |  |
| AVRISP kit |  |
| Power supply | External, 13V (Powered with a current-limited supply (about 150) mA. to prevent accidental damage |
| AVRISP connected to PC and AVR Board | Connect USB cable (cable provided) to PC and to AVRISP box (grey box) |
| Test program | LED_Chase.hex |

## 2	Software set up

- Download and install the latest version of AVR studio from

http://www.atmel.com/

- Open AVR studio Software

- Cancel the pop-up window to open any project / files

- Click on the AVR programming symbol

- Select “ATmega324P” device from “Device” pop-down window

## 3	Test Procedure

- Apply power to the test subject EB019

- Connect AVRISP to J3 of test subject

-     Connect grey header socket (6-way) to J3 of AVR Board, ensure the cable is placed with the tab to the outside of the AVR board (this orientation of the header is shown on the AVR Board silk screen)

- Press “Fuses” tab and change the extended, high and low values to match the values shown below.

- Press “Program” to send fuse data to the AVR chip

- Select the “Program” tab and click the “…” button in the Flash section as shown below

- Navigate to the program LED_Chase.hex

- Press the program button and wait a few seconds (until AVRISP box LED turns green)

- Plug the EB004 LED boards into each of the 4 ports and make sure that:

  - All of the LEDs light up.

  - Only one LED is on at any one time.