# Testing This Product

The following instructions describe the test procedure for the EB-038-00-1 Quad Relay board.

- System Setup

Multi-programmer board (EB-006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F877A |
| SW1 (Fast/Slow) | Don’t care |
| SW2 (RC/Xtal) | Xtal |
| Xtal frequency | 19.6608MHz |
| Port A |  |
| Port B |  |
| Port C | Quad Relay board EB-038 |
| Port D |  |
| Port E |  |
| Test program | RELAYS_2.HEX |

| EB-038 Options | Setting |
| --- | --- |
| Patch jumper setting | LOW/HIGH nibble |

## Multi-programmer Requirements.

Before starting check

- PIC18F2455/16C745 is inserted into single horizontal 28 pin DIL socket.

- PIC 16F877A is inserted into 40 pin DIL socket.

- Ensure 19.6608MHz crystal is inserted into socket.

- On Multi-programmer place SW2 to XTAL (ie, slide switch towards centre of board).

- Insert USB cable.

- Insert 13.5V power supply.

## Test.

- Connect EB-038-00-1 (Quad Relay Board) to PORTC of the multi-programmer and ensure jumper link is placed  on the LOW nibble of the PATCH system. NB. Ensure jumper connector strips are positioned horizontally across the LOW nibble.

- Connect USB cable to computer.

- Ensure 13.5V power supply is working correctly.

- Connect a wire from the +14V screw terminal of the Multi-programmer to +V terminal of Quad Relay Board.

- Open PPP.

- Open file RELAYS_2.HEX.

- Click on “Configure PICmicro..” icon.  In Select Chip section click on pull down menu and select the PIC16F877A.  This will probably be the bottom choice.  Ensure the chip is displayed in the Select Chip section.

- Click ‘OK’ to close the window and return to PPP.

- Once in PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- LEDS D0 to D4 should illuminate in turn and the corresponding relays should be heard to switch (ie, click).

- Once D4 has illuminated remove the jumper from the LOW nibble and place it on the HIGH nibble.

- Again, LEDS D0 to D4 should illuminate in turn and the corresponding relays should be heard to switch (ie, click).

If all LEDs illuminate and corresponding relays switch on both LOW and HIGH nibble settings then the board passes the test.