# Test Procedure for EB051

The following instructions show the test procedure for EB051 Zigbee E-Block Board.

Items required:

- 13.5V power supply (min 13.25V)

- 1 x Multi programmer board (EB006)

      - With PIC16F877A inserted in 40 pin DIL

- 1 x LCD board (EB005)

- Computer with PPP v3 installed that has a USB connection

- USB cable

- Wire

## Set-up

- Ensure that the Multiprogrammer is in correct configuration

  - Fast mode (SW1 towards the centre of the board)

  - Ensure that a 19.6608MHz crystal is inserted in the Multiprogrammer board

- Insert the LCD board (EB005) into Port B of the Multiprogrammer

- Connect wire from “+V” of LCD board to “+V” of Multiprogrammer

- Program the a PIC16F877A with the test program Zigbee.hex

## TEST

- Insert the Zigbee board into Port C of the Multiprogrammer

- Insert the XBEE module into the socket on the Zigbee board

- Connect wire from “+V” of the Zigbee board to “+V” of Multiprogrammer.

- Make sure jumpers "C" and "1" are selected on the Zigbee board.

- Switch on Power to on the Multiprogrammer.

- The word “Testing” should appear on the LCD screen.

- The LED in position D1 should light and start flashing

- The LCD in should then print Sleep and the LED D1 should remain constant for a few seconds.

If the word OK appears on the LCD and the LED flashed then testing is successful.

Otherwise if an error message is displayed or the LED did not briefly stop flashing then the test has failed.