# Testing This Product

The following instructions describe the test procedure for the EB069-00-1 WLAN board.

- System Setup

Multi-programmer board (EB-006) with:

| EB006 Options | Setting |
| --- | --- |
| Power supply | External, 14V |
| PICmicro device | 16F877A |
| SW1 (Fast/Slow) | Don’t care |
| SW2 (RC/Xtal) | Xtal |
| Xtal frequency | 19.6608MHz |
| Port A |  |
| Port B | LCD Board EB005 |
| Port C | WLAN Board EB069 |
| Port D |  |
| Port E |  |
| Test program | WLAN.HEX |

## 2.   Setup

- Connect EB005-00-1 (LCD board) to PORTB of the Multiprogrammer.

- Ensure EB005 jumper setting is in the default position.

- Connect USB cable to computer and EB006.

- Connect a wire from the +V screw terminal of the Multi-programmer to +V terminal of LCD board.

- Ensure power supply is set to 13.5V and working correctly and connect to EB006.

- Open PPP.

- In PPP open file WLAN.HEX.

- In PPP click on “Send To PICmicro” icon.  A pop up window will inform you of status. If status is ok a “Program Sent And Verified” window will be observed on the screen

- Disconnect PSU From EB006

3.   Testing

- Connect EB069-00-1 (WLAN board) to PORTC of the Multiprogrammer.

- Ensure EB069 jumper setting is in the B position.

- Connect a wire from the +V screw terminal of the Multi-programmer to +V terminal of WLAN board.

- Connect a wire from the +14V screw terminal of the Multi-programmer to +14V of WLAN board.

- Ensure power supply is set to 13.5V and working correctly and connect to EB006.

- Wait for the LCD to display Startup OK.

- The network “Flowcode” should appear on a wireless list of a enabled laptop or mobile phone

- Connect to the Flowcode network, you may get a warning about connecting to an un-secure network, if you do then click yes.

- Open an internet browser and in the options check that the proxy setting is set to no setting.

- Type in the address bar the following URL: 192.168.0.2:5000

- Hit return or click the go button

- The message Hello World should appear in the web browser.

- The LCD should display that a page has been requested and served out.

- Disconnect wires from EB069 and unplug PSU From EB006

If the web page data is appearing on the browser and the LCD is displaying that a page has been requested then the board has passed testing.

Additional Notes:

To anyone having problems with the WLAN board. 

Basically the WLAN board features a WIZ610wi module that the microcontroller communicates with. The problem with the modules seems to be in getting them to reply to commands being sent by the microcontroller. A reason for this is that the module uses the UART pins for commands and wireless data and the module seems to sometimes get stuck in data mode. Once the module has replied to incoming commands then the two devices run hand in hand using the component code.

Another issue is that the board requires two wired connections from the programmer. +V and VPWR. You must ensure both these connections are wired into the module the correct way around before powering up the board. Be very careful with the VPWR cable as if this shorts against anything on the E-blocks then it could potentially cause all kinds of problems such as killing your WLAN module.

The first step to take when troubleshooting the board is to update to the latest version of Flowcode. Flowcode PIC v5.2 will be available shortly and this includes the latest recommended fix for the WLAN component C code.

The WLAN component uses a few levels of the stack so you may need to enable the software call stack to fully support this device on a 8-bit PIC. To do this select the Chip -> Compiler options menu in Flowcode and click on the Linker tab. Add the following section to the end of the linker parameters text field.

-swcs 6 2

The WLAN component also produces a warning during compilation saying that the delay is used in the main and in the interrupt. This is not the case as the delay in the interrupt code is never called so this warning can be safely ignored.

There are two variations of WLAN E-blocks at the moment. One module with the v1.1.29 firmware, and one with the v1.2.3 firmware, which will be marked by a sticker on the underside of the module. The v5.2 component C file has been tested with both firmware revisions but the 1.2.3 firmware performs better. You can update the firmware on the earlier models by using a laptop or smart phone to connect to the module and visiting the WLAN setup wizard. The firmware can downloaded from the WIZ610wi downloads page on the Wiznet website.

v1.2.3 FW takes approx 30 seconds to initialise from cold start
v1.1.29 FW takes approx 60-90 seconds to initialise from cold start

If all else fails then you can attempt a factory reset of the WLAN settings using the code customisation feature in Flowcode and un-commenting the delay in the init function code. This can be useful if you want to be able to login to modify the loaded firmware version. With the v1.1.29 firmware I found that you may have to up the delay from 4 seconds to 30 seconds to ensure the factory reset is performed.